SMART POWER PLUS — RGB LED INDICATION LOGIC

## Power ON and Hotspot (Wi-Fi Provisioning Ready)

- When the device is powered ON using 230V AC supply:

  - The MCU receives supply and starts booting.

  - The device automatically starts its own Wi-Fi Hotspot (Access Point mode).

  - The device is now ready for Wi-Fi provisioning using the companion mobile app.

- LED Behaviour:

  - The RGB LED blinks Green every 1 second.

  - This blinking pattern indicates:

    - The device is active.

    - Hotspot is running and waiting for Wi-Fi provisioning.

    - User should now connect using the mobile app.

## Wi-Fi Provisioning and Connection Status

- The user opens the mobile provisioning app and scans for available SPP devices (e.g., SPPA00001, SPPB00016) via SoftAP.

- The user selects the desired SPP device from the scan results and proceeds to the provisioning screen.

- The app displays available Local Wi-Fi networks (SSIDs); the user selects a network or enters it manually.

- The user enters the Wi-Fi password and starts the provisioning process.

- The device attempts connection to the selected local Wi-Fi network and reports provisioning status back to the app.

- If Wi-Fi connection is successful:

  - The LED pattern depends on the Wi-Fi signal strength:

    - Good signal (≥ 60% strength):

      - RGB LED turns Green solid ON (no blinking).

      - Indicates stable Wi-Fi connection with good strength.

    - Weak signal (< 60% strength):

      - RGB LED blinks Green and Red alternately, with each colour appearing every 1 second.

      - This warns the user that the Wi-Fi connection is weak, which could affect data transmission.

## Data Transmission to AWS IoT Core

- The device starts sending data to AWS IoT Core at the configured frequency.

- If data is successfully sent to the cloud:

  - The LED pattern depends on the Wi-Fi strength:

    - Good Wi-Fi signal (≥ 60%) + successful data push:

      - RGB LED blinks Green and Blue alternately (each colour appears every 1 second).

      - Indicates successful cloud communication and strong Wi-Fi.

    - Weak Wi-Fi signal (< 60%) + successful data push:

      - RGB LED blinks in a sequence: Green → Red → Blue, with each colour appearing for 1 second before changing to the next.

      - Indicates successful cloud communication but warns about weak Wi-Fi strength.

- This ensures the user is aware of both the data transmission status and Wi-Fi quality.

## Wi-Fi Disconnection

- At any point during operation:

  - If the Wi-Fi connection is lost (disconnected from the router or access point):

    - RGB LED blinks Red continuously, every 1 second.

    - This indicates:

      - The device is offline and not connected to Wi-Fi.

      - No data is being sent to the cloud.

- Once Wi-Fi reconnects:

  - The LED pattern updates according to the current Wi-Fi strength and data transmission status.

## Quick LED Pattern Reference

| Condition | LED Pattern |
| --- | --- |
| Power ON + Hotspot active | Green blink (1 sec interval) |
| Wi-Fi connected (≥ 60% signal) | Green solid ON |
| Wi-Fi connected (< 60% signal) | Green & Red blink alternately (1 sec interval) |
| Data push success + good Wi-Fi | Green & Blue blink alternately (1 sec interval) |
| Data push success + weak Wi-Fi | Green → Red → Blue blink sequence (1 sec each) |
| Wi-Fi disconnected | Red blink (1 sec interval) |

## How this helps the user

- The LED patterns provide immediate visual feedback:

  - On device readiness (hotspot, provisioning).

  - Wi-Fi connection quality.

  - Data push status to AWS IoT Core.

  - Alerts for disconnection or weak connectivity.